Energy Modelling

Energy Transition Scenario Framework

Exiobase-Based Multi-Regional Input-Output Analysis

April 2026

Geographic Coverage

| Type | Geography | Exiobase Code |
| --- | --- | --- |
| Country | South Africa | ZA |
| Country | India | IN |
| Country | Brazil | BR |
| Region | Rest of Africa | WF (Africa aggregate) |
| Region | Rest of Middle East | WM (MENA aggregate) |

Two Scenarios Per Geography

| # | Scenario | Description |
| --- | --- | --- |
| S1 | Full Decarbonization | 100% renewable electricity by 2050 — upper-bound transition shock replacing all fossil generation |
| S2 | National Policy Target | Country/region-specific targets from the most recent NDCs, national energy plans, and JETP commitments (2023–2025) |

# Table of Contents

Table of Contents	1

1. Introduction and Modelling Context	1

1.1 The MARIO Model	1

1.2 Exiobase 3 and Energy Sectors	1

1.3 Shock Implementation Logic in MARIO	1

2. South Africa	1

2.1 Policy Context	1

2.2 Scenario S1 — Full Decarbonization (ZA-S1)	1

2.3 Scenario S2 — National Policy Target (ZA-S2)	1

3. India	1

3.1 Policy Context	1

3.2 Scenario S1 — Full Decarbonization (IN-S1)	1

3.3 Scenario S2 — National Policy Target (IN-S2)	1

4. Brazil	1

4.1 Policy Context	1

4.2 Scenario S1 — Full Decarbonization (BR-S1)	1

4.3 Scenario S2 — National Policy Target (BR-S2)	1

5. Rest of Africa (WF Region)	1

5.1 Policy Context	1

5.2 Scenario S1 — Full Decarbonization (WF-S1)	1

5.3 Scenario S2 — National Policy Target (WF-S2)	1

6. Rest of Middle East (WM Region)	1

6.1 Policy Context	1

6.2 Scenario S1 — Full Decarbonization (WM-S1)	1

6.3 Scenario S2 — National Policy Target (WM-S2)	1

7. MARIO Model Implementation Guide	1

7.1 Shock File Structure	1

7.2 Shock Parameter Conventions	1

7.3 Calibration Approach	1

7.4 Scenario Summary Table	1

7.5 Key Modelling Assumptions and Limitations	1

8. Key Sources and References	1

South Africa	1

India	1

Brazil	1

Rest of Africa	1

Rest of Middle East	1

Modelling Methodology	1

# 1. Introduction and Modelling Context

This document defines the scenario framework for the MARIO (Multi-Regional Input-Output) model, implemented using the Exiobase 3 multi-regional supply and use table (MSUT). The scenarios are designed to quantify the economic and environmental consequences of energy sector decarbonization across five strategically selected geographies: South Africa, India, Brazil, Rest of Africa, and Rest of the Middle East.

Each geography is assigned two scenarios: (i) a stylized full decarbonization shock representing an upper-bound 100% renewable transition, and (ii) a policy-aligned scenario grounded in the most recent national and regional climate commitments (2023–2025). Both scenarios are translated into MARIO-compatible "shocks" — numerical adjustments to the technical coefficient matrix (A-matrix), satellite extension intensities, and final demand vectors within Exiobase.

## 1.1 The MARIO Model

MARIO (Modelling And Research on Input-Output) is an open-source Python library developed for multi-regional input-output analysis. It enables scenario analysis through the implementation of "shocks" — perturbations to the baseline economic structure that represent structural changes in technology, policy, or behaviour. The model operates on the Leontief demand-pull or Ghosh supply-push framework and supports satellite account analysis for environmental pressures (CO2 emissions, energy use, material inputs).

## 1.2 Exiobase 3 and Energy Sectors

Exiobase 3 covers 49 countries and 5 rest-of-world regions, with 163 industries and 200 products per region. The energy sectors most relevant to this analysis include:

- Electricity by coal

- Electricity by gas

- Electricity by nuclear

- Electricity by hydro

- Electricity by wind

- Electricity by solar photovoltaic

- Electricity by biomass and waste

- Electricity by oil

- Electricity nec (other)

Scenarios are implemented by modifying the inter-industry technical coefficients for these sectors (reducing fossil-based shares and increasing renewable shares) and adjusting the CO2/energy satellite extensions accordingly.

## 1.3 Shock Implementation Logic in MARIO

In MARIO, each scenario is defined by a shock file (Excel/CSV) that specifies changes to model parameters. The key shock types used in this framework are:

| Shock Type | MARIO Parameter | Description |
| --- | --- | --- |
| Technology shift | Z matrix / A matrix | Reallocate fossil electricity production to renewable alternatives (proportional substitution) |
| Emission intensity | Satellite (E matrix) | Reduce CO2 emission coefficients for electricity sectors in proportion to fossil phase-out |
| Capacity addition | Final demand (Y) | Increase capital investment demand for renewable generation equipment and grid infrastructure |
| Fuel use intensity | A matrix (energy rows) | Reduce coal/gas/oil input per unit of electricity produced in fossil sectors being phased down |

# 2. South Africa

## 2.1 Policy Context

South Africa is the world's most carbon-intensive major economy relative to GDP, and the African continent's largest emitter. Its electricity system is dominated by coal (approximately 83% of generation in 2023), operated predominantly by the state utility Eskom. South Africa has committed to a significant energy transition through its Integrated Resource Plan (IRP 2023), the landmark Climate Change Act (2024), and the Just Energy Transition Partnership (JETP) with a $9.3 billion international funding package.

| Policy Document | Key Targets |
| --- | --- |
| Second NDC (Oct 2025) | 350–420 MtCO2e by 2030; 320–380 MtCO2e by 2035 |
| IRP 2023 | 22% renewable electricity by 2030; 29.3 GW new capacity; no new coal beyond current construction |
| Climate Change Act 2024 | Legally binding net-zero 2050; peak emissions 2020–2025; decline from 2036 |
| JETP Commitment | $9.3 billion from G7+EU; coal phase-down post-2035; renewable capacity prioritisation |
| Baseline (2023) | Coal: ~83% electricity; Renewables: ~4.4% total energy supply; total installed: ~51 GW |

## 2.2 Scenario S1 — Full Decarbonization (ZA-S1)

This scenario represents a stylized upper-bound shock in which 100% of electricity generation shifts to renewables by 2050. Coal, natural gas, and oil-based electricity production are completely phased out. This scenario is intentionally extreme and serves as a structural sensitivity benchmark for the model.

| ZA-S1 — South Africa Full Decarbonization |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Fossil electricity share | 50% → 0% (partial by 2030) | 0% (complete phase-out) |
| Renewable electricity share | 22% → 60% by 2030 | 100% by 2050 |
| Coal electricity coefficient | Reduce by 50% in A-matrix | Reduce to 0 in A-matrix |
| Gas electricity coefficient | Reduce by 30% | Reduce to 0 |
| Wind/Solar coefficient | +0.38 share reallocation | +1.0 (absorb all fossil share) |
| CO2 emission intensity (elec.) | Reduce by ~50% | Reduce to 0 |
| Shock kind in MARIO | Percentage / New Value | New Value = 0 (fossils) |

MARIO Implementation Notes: Apply negative percentage shocks to coal, gas, and oil electricity rows in the A-matrix for the ZA region. Apply corresponding positive shocks to wind, solar PV, and hydro rows. Scale satellite CO2 extensions proportionally. Use 2030 as the intermediate shock year and 2050 as the terminal year.

## 2.3 Scenario S2 — National Policy Target (ZA-S2)

This scenario follows South Africa's actual policy commitments under the IRP 2023, Second NDC (2025), and JETP. It reflects the government's endorsed trajectory balancing socioeconomic constraints with climate goals, including continued coal use through 2035 and a gas bridge strategy.

| ZA-S2 — South Africa Policy Target |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Renewable electricity share | 22% by 2030 (IRP 2023) | Net-zero electricity by 2050 |
| Coal electricity share | ~70% in 2025 → ~55% by 2030 | Phase-out post 2035; ~0% by 2050 |
| New gas capacity | +7 GW gas by 2030 (IRP 2023) | Gradual phase-down 2035–2050 |
| Solar/Wind addition (GW) | +8.1–13.8 GW by 2030 | +full capacity for net-zero |
| GHG emissions target | 350–420 MtCO2e by 2030 | 212–428 MtCO2e by 2050 (Act) |
| Coal A-matrix coefficient | Reduce by ~15% by 2030 | Reduce to 0 by 2050 |
| Shock kind in MARIO | Percentage (moderate) | New Value (net-zero state) |

Policy sources: Integrated Resource Plan 2023 (Dept. of Energy, ZA); Second NDC submitted to UNFCCC October 2025; Climate Change Act 22 of 2024; JETP Investment Plan 2023. Quantitative shock values should be calibrated against Exiobase 2021 baseline output shares for ZA energy sectors.

# 3. India

## 3.1 Policy Context

India is the world's third-largest emitter and one of the fastest-growing energy markets globally. Its electricity system is undergoing rapid transformation: as of mid-2025, India has already achieved its 2030 NDC target of 50% non-fossil installed capacity — five years ahead of schedule. Despite this, India continues to expand coal capacity to meet surging electricity demand. India's net-zero commitment is set for 2070, giving a longer transition horizon than developed nations.

| Policy Document | Key Targets |
| --- | --- |
| Updated NDC (Aug 2022) | 45% emissions intensity reduction from 2005 by 2030; 50% non-fossil capacity by 2030 (ACHIEVED 2025) |
| Panchamrit (COP26) | 500 GW non-fossil capacity; 1 BtCO2 cumulative reduction; 50% renewable energy share; net-zero 2070 |
| National Electricity Plan 2023 | 596 GW renewable by 2031-32 (66% of 900 GW total); Solar 364 GW, Wind 122 GW |
| Net-Zero Target | 2070 (announced COP26 by PM Modi); renewables 65% of energy mix by 2070 |
| Baseline (2023) | Coal: ~59% of installed capacity; Renewables: ~168 GW (44%); total installed ~450 GW |

## 3.2 Scenario S1 — Full Decarbonization (IN-S1)

Upper-bound scenario: 100% renewable electricity by 2070, aligned with India's net-zero year. Gradual linear ramp-up of renewables and phase-out of coal and gas electricity in the A-matrix.

| IN-S1 — India Full Decarbonization |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Fossil electricity share | 50% → 25% by 2030 | 0% by 2070 (net-zero year) |
| Renewable electricity share | 50% → 75% by 2030 | 100% by 2070 |
| Coal A-matrix coefficient | Reduce by 25% (2030) | Reduce to 0 (2070) |
| Solar PV coefficient | Increase by +30% | Scale to 65% of total mix |
| Wind coefficient | Increase by +15% | Scale to 25% of total mix |
| CO2 emission intensity | Reduce by ~45% (NDC target) | Reduce to 0 |
| Shock kind in MARIO | Percentage (large) | New Value = 0 (fossils) |

## 3.3 Scenario S2 — National Policy Target (IN-S2)

Follows India's NEP 2023 trajectory with 500 GW non-fossil capacity by 2030, while maintaining coal as a residual source through 2031-32. The long-run shock extends to 2070 aligned with the net-zero target.

| IN-S2 — India Policy Target |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Non-fossil capacity | 500 GW by 2030 (Panchamrit) | ~1,200–1,500 GW by 2070 |
| Solar capacity | 364 GW by 2031-32 (NEP 2023) | Dominant source by 2070 |
| Wind capacity | 122 GW by 2031-32 (NEP 2023) | Second largest source |
| Coal share (capacity) | ~32% by 2031-32 (down from 59%) | ~4% by 2070 (LT-LEDS) |
| Emissions intensity (GDP) | -45% by 2030 vs. 2005 (NDC) | Net-zero by 2070 |
| Coal A-matrix coefficient | Reduce by ~35% by 2030 | Reduce to near-zero by 2070 |
| Shock kind in MARIO | Percentage (moderate-large) | New Value (near-zero fossils) |

Policy sources: Government of India updated NDC August 2022; National Electricity Plan 2023 (Central Electricity Authority); PM Modi COP26 Panchamrit announcement; Long-Term Strategy for Low Carbon Development (LT-LEDS, COP27).

# 4. Brazil

## 4.1 Policy Context

Brazil is a unique case among major economies: it already generates approximately 85-88% of its electricity from renewable sources, dominated by large hydropower. Under President Lula, Brazil has significantly strengthened its climate ambitions, bringing forward its net-zero target from 2060 to 2050 and submitting a highly ambitious NDC 3.0 (November 2024) committing to 59-67% total emissions reduction by 2035 vs. 2005. Brazil's decarbonization challenge lies not in electricity but in deforestation (LULUCF), agriculture, and hard-to-abate industrial sectors.

| Policy Document | Key Targets |
| --- | --- |
| NDC 3.0 (Nov 2024, COP29) | 59–67% emissions reduction by 2035 vs. 2005; climate neutrality by 2050 |
| Previous NDC (2023) | -48% by 2025, -53% by 2030 vs. 2005; zero illegal deforestation by 2030 |
| Renewable Energy Target | 90% electricity from renewables by 2030 (already at 88% in 2024); 45-50% of total energy |
| Nova Industria Brasil (2024) | R$300 billion (2024-2026) for industrial decarbonization; green hydrogen, biofuels, bioeconomy |
| Deforestation | Zero illegal deforestation by 2030 (Amazon + Cerrado); PPCDAm plan 2023 |
| Baseline (2023) | Hydro: ~65%; Wind: ~12%; Solar: ~5%; Biomass: ~8%; Fossil: ~10% of electricity |

## 4.2 Scenario S1 — Full Decarbonization (BR-S1)

Given Brazil's already high renewable electricity share, this scenario focuses on the complete elimination of fossil-based generation (natural gas peakers, oil-based backup), and the decarbonization of heat and transport sectors that feed into the MRIO model. The electricity sector shock is moderate; the primary shock is in natural gas and oil input coefficients.

| BR-S1 — Brazil Full Decarbonization |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Fossil electricity share | ~10% → 0% by 2030 (aggressive) | 0% (complete phase-out) |
| Renewable electricity share | 88% → 100% by 2030 | 100% maintained |
| Gas electricity coefficient | Reduce to 0 in A-matrix by 2030 | 0 (maintained) |
| Oil electricity coefficient | Reduce to 0 in A-matrix by 2030 | 0 (maintained) |
| Hydro + Wind + Solar | Proportional reallocation to 100% | Maintained at 100% |
| CO2 emission intensity (elec.) | Reduce to near-zero by 2030 | 0 by 2050 |
| Shock kind in MARIO | New Value = 0 (gas/oil electricity) | New Value = 0 |

## 4.3 Scenario S2 — National Policy Target (BR-S2)

Follows Brazil's actual policy trajectory: 90% renewables in electricity by 2030 (already nearly achieved), with deeper industrial and transport decarbonization through the Nova Industria Brasil program. The primary MARIO shocks involve input coefficients for fossil fuels in industrial processes, not just power generation.

| BR-S2 — Brazil Policy Target |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Renewable electricity | 90% by 2030 (target, ~achieved) | 95%+ by 2050 |
| Total renewable energy share | 45–50% of primary energy by 2030 | Climate neutral by 2050 |
| GHG emissions target | -53% by 2030 vs. 2005 | -59 to -67% by 2035; net-zero 2050 |
| Deforestation (LULUCF) | Zero illegal deforestation 2030 | LULUCF carbon sink by 2050 |
| Industrial decarbonization | R$300B Nova Industria 2024-2026 | Full hard-to-abate coverage |
| Gas electricity coefficient | Reduce by ~80% by 2030 | Reduce to ~0 by 2050 |
| Shock kind in MARIO | Percentage (electricity minor) | New Value (LULUCF shock large) |

Policy sources: Brazil NDC 3.0 submitted UNFCCC November 2024 (COP29); Updated NDC 2023; Nova Industria Brasil launch January 2024; PPCDAm (Action Plan for Prevention and Combat of Deforestation in the Amazon) 2023 revision; IEA Brazil Energy Country Profile 2024.

# 5. Rest of Africa (WF Region)

## 5.1 Policy Context

The 'Rest of Africa' Exiobase region aggregates sub-Saharan Africa (excluding South Africa) and North African countries not individually represented. This region faces a dual challenge: expanding energy access for approximately 600 million people without electricity while meeting climate commitments. The African Union's Agenda 2063 and the Nairobi Declaration (2023) set the continental framework, while AREI (Africa Renewable Energy Initiative) targets 300 GW of renewable capacity by 2030.

| Policy Document | Key Targets |
| --- | --- |
| Nairobi Declaration (2023) | 300 GW renewable capacity by 2030 (from 56 GW in 2022); renewable financing to 20% |
| AREI (Africa RE Initiative) | 300 GW renewable by 2030; 10 GW Phase I completed; $600B investment needed |
| Mission 300 | Electricity for 300 million people by 2030 (World Bank + AfDB); 90M per year required |
| IEA Africa Energy Outlook 2022 | 80%+ new power from renewables by 2030; 33% energy demand growth 2020-2030 |
| Morocco NDC (2021) | 18.3% unconditional / 45.5% conditional GHG reduction by 2030 vs. BAU |
| Baseline (2022) | 55%+ already from renewables (large hydro dominant); fossil 45%; 56 GW renewable installed |

## 5.2 Scenario S1 — Full Decarbonization (WF-S1)

Upper-bound scenario: 100% renewable electricity region-wide by 2050. Given the existing high renewable share in some sub-regions (hydro-rich East Africa) and fossil dependence in others (Nigeria, North Africa), the shock is a full reallocation of fossil-based generation to renewables across the aggregated region.

| WF-S1 — Rest of Africa Full Decarbonization |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Fossil electricity share | ~45% → 20% by 2030 | 0% by 2050 |
| Renewable electricity share | 55% → 80% by 2030 | 100% by 2050 |
| Coal coefficient (A-matrix) | Reduce by 40% | Reduce to 0 |
| Gas coefficient (A-matrix) | Reduce by 35% | Reduce to 0 |
| Oil/diesel generation | Reduce by 50% | Reduce to 0 |
| Solar PV + Wind coefficient | +0.45 share reallocation | +1.0 (absorb all fossil) |
| CO2 emission intensity (elec.) | Reduce by ~40% | Reduce to 0 |

## 5.3 Scenario S2 — National Policy Target (WF-S2)

Follows the continental policy framework: AREI 300 GW target by 2030 (representing a 5x increase from 2022 levels), while simultaneously expanding energy access under Mission 300. The shock reflects both supply-side renewable capacity addition and demand-side growth from 300 million newly electrified consumers.

| WF-S2 — Rest of Africa Policy Target |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Renewable capacity | 300 GW by 2030 (AREI/Nairobi) | 600+ GW by 2050 (estimated) |
| Energy access expansion | +300M people electrified by 2030 | Universal access by 2040 |
| Renewable electricity share | 80% of new additions from RE | 85%+ of total generation |
| Fossil share reduction | 45% → 30% by 2030 | 15% by 2050 (gas bridge) |
| Renewable A-matrix coefficient | +435% capacity increase (2022-2030) | Sustained expansion |
| Final demand (Y) shock | +electricity demand from 300M new consumers | Full access demand |
| Shock kind in MARIO | Percentage + demand expansion | Combined supply-demand shock |

Policy sources: Nairobi Declaration on Climate Change, Africa Climate Summit September 2023; AREI (Africa Renewable Energy Initiative) Progress Report 2023; IEA Africa Energy Outlook 2022; World Bank Mission 300 Programme 2024; African Development Bank Energy Access targets 2024.

# 6. Rest of Middle East (WM Region)

## 6.1 Policy Context

The 'Rest of Middle East' Exiobase region covers the Gulf Cooperation Council (GCC) states, the Levant, and other MENA countries not individually listed in Exiobase (i.e., excluding countries like Iran, Turkey, which have their own Exiobase entries). The region is characterized by near-total dependence on natural gas for electricity generation (70%+ of MENA electricity) and a dual agenda: maintaining hydrocarbon export revenues while transitioning domestic energy systems. Saudi Arabia's Vision 2030, UAE's Net Zero 2050 strategy, and Qatar's National Vision 2030 define the regional ambition level. COP28 (Dubai, 2023) was a milestone event for the region.

| Policy Document | Key Targets |
| --- | --- |
| Saudi Vision 2030 (Energy) | 50% renewable electricity by 2030; 130 GW renewable (58.7 GW solar, 40 GW wind) |
| UAE Net Zero 2050 Strategy | 32% clean energy by 2030; 0.27 kgCO2/kWh grid intensity; net-zero by 2050 |
| Qatar National Vision 2030 | 18% renewable by 2030 (from 5%); 4 GW renewable capacity; 25% GHG reduction |
| Oman Vision 2040 | 30% renewable power by 2030; 35-39% by 2040; net-zero 2050 |
| Kuwait | 30% renewable by 2030; 50% by 2050; net-zero 2060 |
| IRENA MENA Vision 2050 | 200 GW solar PV by 2035 (10x increase); 53% renewables in power sector by 2050 |
| COP28 UAE Consensus (Dec 2023) | Transition away from fossil fuels; triple renewables globally by 2030; $700M L&D Fund |
| Baseline (2023) | Gas: ~70% electricity; Oil: ~20%; Renewables: ~6-10%; Total installed ~200 GW (est.) |

## 6.2 Scenario S1 — Full Decarbonization (WM-S1)

Upper-bound scenario representing a complete phase-out of gas and oil-based electricity, replaced entirely by solar PV and wind. This is the most radical departure from the current energy mix among all five geographies, given the region's near-total fossil dependence in electricity generation.

| WM-S1 — Rest of Middle East Full Decarbonization |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Gas electricity share | ~70% → 35% by 2030 | 0% by 2050 |
| Oil electricity share | ~20% → 5% by 2030 | 0% by 2050 |
| Solar PV coefficient | +55% reallocation by 2030 | Absorbs 80% of total mix |
| Wind coefficient | +15% reallocation by 2030 | Absorbs 20% of total mix |
| Renewable electricity share | ~10% → 60% by 2030 | 100% by 2050 |
| CO2 emission intensity (elec.) | Reduce by ~55% | Reduce to 0 |
| Shock kind in MARIO | Percentage (very large) | New Value = 0 (gas/oil) |

## 6.3 Scenario S2 — National Policy Target (WM-S2)

Follows a weighted average of the GCC states' official policy targets, anchored to Saudi Arabia's 50% renewables by 2030 (as the dominant regional economy), UAE's net-zero 2050, and the regional IRENA trajectory. The scenario includes a significant residual gas role as a transition fuel, consistent with regional plans.

| WM-S2 — Rest of Middle East Policy Target |  |  |
| --- | --- | --- |
| Parameter | 2030 Target | 2050 Target |
| Renewable electricity share | ~35% by 2030 (weighted avg) | 53% by 2050 (IRENA MENA) |
| Solar PV capacity | +130 GW total (Saudi: 58.7 GW solar) | 200 GW+ by 2035 (IRENA) |
| Gas electricity share | ~70% → ~55% by 2030 | ~35% by 2050 (transition fuel) |
| Net-zero commitments | UAE/Oman: 2050; SA/Kuwait: 2060 | Region net-zero ~2055 avg |
| Grid carbon intensity | 0.27 kgCO2/kWh (UAE 2030) | Near-zero by 2050 |
| Gas A-matrix coefficient | Reduce by ~20% | Reduce by ~55% |
| Solar coefficient | Increase by +30% (A-matrix) | Increase to 40-50% share |
| Shock kind in MARIO | Percentage (moderate) | Percentage / New Value mix |

Policy sources: Saudi Arabia Ministry of Energy Vision 2030 renewable targets; UAE Net Zero 2050 National Strategy; Qatar National Vision 2030 Energy & Environment chapter; Oman Authority for Public Services and Regulatory Affairs 2040 plan; Kuwait Ministry of Electricity 2030 targets; IRENA MENA Renewable Energy Outlook 2023; COP28 UAE Consensus December 2023.

# 7. MARIO Model Implementation Guide

## 7.1 Shock File Structure

Each scenario is implemented in MARIO through a shock Excel file (.xlsx) structured with the following sheets. The companion Excel file ('MARIO_Scenario_Shocks.xlsx') provides the ready-to-use template populated with the quantitative parameters derived in this report.

| Sheet Name | MARIO Parameter | Content Description |
| --- | --- | --- |
| Z_shocks / A_shocks | Z matrix (flows) or A matrix (coefficients) | Inter-industry coefficient changes: fossil electricity rows decrease, renewable rows increase for the target region |
| E_shocks | Satellite extension (E) matrix | CO2 emission intensity changes per unit of electricity produced (kg CO2 / EUR output) |
| Y_shocks | Final demand (Y) vector | Demand-side shocks: capital investment in renewables, energy access expansion (WF region) |
| Metadata | N/A | Scenario ID, geography, year, shock type, source citation, and model notes |

## 7.2 Shock Parameter Conventions

The following conventions apply to all shock files in this framework:

- Shock magnitude is expressed as a multiplier (1.0 = no change; 0.0 = complete elimination; 2.0 = doubling).

- Percentage shocks are expressed as decimal changes (e.g., -0.15 means a 15% reduction).

- New Value shocks replace the existing coefficient entirely (used for complete phase-outs).

- All shocks are applied to Exiobase 2021 as the base year. Model runs should iterate for 2025, 2030, 2035, 2040, 2045, and 2050 time steps.

- Fossil electricity sectors being phased out: 'Electricity by coal', 'Electricity by gas', 'Electricity by oil'.

- Renewable electricity sectors receiving reallocated shares: 'Electricity by wind', 'Electricity by solar photovoltaic', 'Electricity by hydro', 'Electricity by biomass'.

- CO2 satellite extension row labels in Exiobase: CO2 - combustion - air (direct emissions); CO2 - non-combustion - Cement production - air; CO2 - waste - biogenic - air.

## 7.3 Calibration Approach

Before running the full scenario, the following calibration steps are recommended:

| Step | Action | Notes |
| --- | --- | --- |
| 1 | Extract baseline energy shares | Query Exiobase 2021 Z-matrix output shares for each electricity sector in each target region |
| 2 | Compute fossil share | Sum coal+gas+oil electricity as % of total electricity output for each region |
| 3 | Derive shock multipliers | Shock = (target share / baseline share); if target = 0, use New Value = 0 mode |
| 4 | Apply reallocation | Reduction in fossil coefficients must sum to increase in renewable coefficients (row sums preserved) |
| 5 | Validate satellite accounts | Verify CO2 intensity reductions are proportional to fossil share reduction |
| 6 | Run model and check Leontief inverse | Ensure no negative outputs; check Leontief stability conditions |

## 7.4 Scenario Summary Table

Complete cross-geography scenario overview for quick reference:

| ID | Geography | RE Share 2030 | RE Share 2050 | Fossil Δ 2030 | Net-Zero Year | Shock Type |
| --- | --- | --- | --- | --- | --- | --- |
| ZA-S1 | South Africa | 60% | 100% | -50% | 2050 | Full |
| ZA-S2 | South Africa | 22% | 100%* | -15% | 2050 | Policy |
| IN-S1 | India | 75% | 100% | -25% | 2070 | Full |
| IN-S2 | India | 66% | ~96%* | -35% | 2070 | Policy |
| BR-S1 | Brazil | 100% | 100% | -100%* | 2050 | Full |
| BR-S2 | Brazil | 90% | 95%+ | -80% | 2050 | Policy |
| WF-S1 | Rest Africa | 80% | 100% | -40% | 2050 | Full |
| WF-S2 | Rest Africa | 80%** | 85%+ | -30% | 2060* | Policy |
| WM-S1 | Rest M.East | 60% | 100% | -55% | 2050 | Full |
| WM-S2 | Rest M.East | ~35% | 53% | -20% | ~2055 | Policy |

* Net-zero for electricity sector specifically; ** new generation only; * BR-S1 fossil electricity is ~10% of total, hence -100% of that residual fossil share

## 7.5 Key Modelling Assumptions and Limitations

- Exiobase 2021 is used as the baseline year. Structural changes since 2021 are not reflected in the baseline A-matrix.

- Shocks assume partial equilibrium — second-order supply chain effects are captured by the Leontief inverse but behavioural responses (price elasticities) are not modelled.

- The Rest of Africa (WF) and Rest of Middle East (WM) Exiobase regions are heterogeneous aggregates. Country-level policies are mapped to the aggregate region using GDP-weighted averages where possible.

- Energy capacity targets (GW) are translated to production share changes using assumed capacity factors: solar 20%, wind 30%, hydro 45%, coal 70%, gas 55%.

- LULUCF shocks for Brazil are excluded from this electricity-focused scenario framework but should be incorporated in a broader land-use model extension.

- Hydrogen production shocks (relevant for Saudi Arabia, UAE, and Brazil) are noted as a future model extension beyond this baseline scenario set.

# 8. Key Sources and References

## South Africa

- South Africa Second Nationally Determined Contribution, UNFCCC, October 2025.

- Integrated Resource Plan 2023, Department of Mineral Resources and Energy, Republic of South Africa.

- Climate Change Act (Act 22 of 2024), Government Gazette No. 50966, 23 July 2024.

- Just Energy Transition Investment Plan (JETP-IP), Presidential Climate Commission, 2023.

- South Africa Energy Outlook, International Energy Agency, 2023.

## India

- India Updated Nationally Determined Contribution, Ministry of Environment, Forest and Climate Change, August 2022.

- National Electricity Plan 2023, Central Electricity Authority, Government of India.

- Panchamrit — Five Nectar Elements for India's Climate Action, COP26, November 2021.

- Long-Term Low Carbon Development Strategy (LT-LEDS), Government of India, COP27 2022.

## Brazil

- Brazil Third Nationally Determined Contribution (NDC 3.0), UNFCCC, November 2024 (COP29).

- Nova Industria Brasil Programme, Ministry of Development, Industry, Commerce and Services, January 2024.

- PPCDAm — Action Plan for Prevention and Combat of Deforestation in the Amazon, 2023 revision.

- Brazil Energy Policy Review, International Energy Agency, 2024.

## Rest of Africa

- African Leaders Nairobi Declaration on Climate Change, Africa Climate Summit, September 2023.

- Africa Renewable Energy Initiative (AREI) Progress Report, African Union Commission, 2023.

- Africa Energy Outlook 2022, International Energy Agency (with African Union Commission).

- Mission 300 Africa Energy Access Programme, World Bank Group and African Development Bank, 2024.

## Rest of Middle East

- Saudi Arabia Vision 2030 — Renewable Energy Targets, Ministry of Energy, Kingdom of Saudi Arabia, 2023.

- UAE Net Zero 2050 National Strategy, UAE Government, 2021 (reaffirmed COP28 2023).

- Qatar National Vision 2030 — Energy and Environment, Ministry of Planning and Statistics, Qatar.

- Oman Vision 2040 Energy Sector Targets, Authority for Public Services and Regulatory Affairs, Oman.

- The UAE Consensus — COP28 Global Stocktake Outcome, UNFCCC, December 2023.

- MENA Renewable Energy Outlook, International Renewable Energy Agency (IRENA), 2023.

## Modelling Methodology

- Cazcarro, I. et al., MARIO: Multi-Regional Input-Output python library, Journal of Open Source Software, 2022.

- Exiobase 3 — A Detailed Environmentally Extended Multi-Regional Input-Output Table, Journal of Industrial Ecology, Stadler et al., 2018.

- Wood, R. et al., Global Sustainability Accounting — Developing EXIOBASE for Multi-Regional Footprint Analysis, Sustainability, 2015.